Uniform Spec Sheet

Polo & Crew-Neck Tee  ·  v1.0

Rawinnipa Co., Ltd.  ·  April 2026

This document defines the production specifications for Rawinnipa's corporate uniforms across four design concepts, two garment types, and four approved shirt colors. All measurements are production-ready; all placements are millimeter-accurate.

Refer to the Rawinnipa Brand Guidelines v1.0 for master brand rules. Any deviation from this spec — particularly color, logo size, or placement — must be approved in writing by the brand owner.

# 1.  Program summary

### Garment types

- Polo shirt — knit collar, 3-button placket, short sleeve. Unisex cut, adult sizes XS–3XL.

- Crew-neck tee — rib-knit collar, short sleeve. Unisex cut, adult sizes XS–3XL.

### Shirt colors (approved)

- Deep Black  #0A0A0A — primary uniform color.

- Charcoal  #1A1A1A — alternate uniform color.

- Off White  #F7F7F7 — summer / event use.

- Rawin Green  #3EDC81 — brand-moment color, use sparingly.

### Design concepts

- 01 · Classic — chest-left logo, URL on sleeve, tagline heat-transfer at inner neck. Low-key everyday uniform.

- 02 · Bold Back — small chest logo + large embroidered logo centered on the back, tagline beneath. High visibility, best for events and trade shows.

- 03 · Gradient — signature 3-color gradient stripe across the left shoulder + hem tagline. The most visually distinctive option.

- 04 · Typography — large 'RAWINNIPA' wordmark across the chest with a green accent dot, and a stacked 'BUILD / WHAT / MOVES.' slogan across the back. Bold statement piece.

### Quantity matrix

4 concepts × 2 garments × 4 colors = 32 approved SKUs. Front + back views documented individually; mockup library contains 64 PNGs plus 8 concept grids.

# 2.  Color standards

Shirt dye colors shall be lab-dipped and matched against the Pantone references below. Substitutions require written brand approval.

### Shirt fabric colors

| Deep Black #0A0A0A Pantone Black 6 C | Charcoal #1A1A1A Pantone Black 7 C | Off White #F7F7F7 Pantone 11-0601 TCX | Rawin Green #3EDC81 Pantone 2256 C |
| --- | --- | --- | --- |

### Logo & print colors

| Deep Teal #23B591 Pantone 339 C | Rawin Green #3EDC81 Pantone 2256 C | Lime Accent #9DC351 Pantone 2285 C |
| --- | --- | --- |

Pantone values are approximate cross-references — final matches to be confirmed via lab dip at the supplier.

### Ink matrix by shirt color

| Shirt color | Logo variant | Text / URL ink | Accent ink |
| --- | --- | --- | --- |
| Deep Black | logo_white.png | White #FFFFFF | Rawin Green #3EDC81 |
| Charcoal | logo_white.png | White #FFFFFF | Rawin Green #3EDC81 |
| Off White | logo_black.png | Deep Black #0A0A0A | Rawin Green #3EDC81 |
| Rawin Green | logo_white.png | White #FFFFFF | White #FFFFFF (no green-on-green) |

# 3.  Fabric & construction

### Polo shirt

| Fabric | Cotton 65% / Polyester 35% piqué knit, 200–220 GSM |
| --- | --- |
| Collar | Self-fabric rib knit, flat-bed, 2×2 rib, color-matched to body |
| Placket | 3-button, self-fabric reinforced, 18 cm length |
| Buttons | Polyester, color-matched to shirt (deep tone) or body-matched, 11 mm dia. |
| Stitching | Single-needle on hems; 3-needle coverstitch on sleeve & bottom hems |
| Sleeve | Short, self-fabric banded hem, 3.5 cm band |
| Side seam | Open side seam, 1.0 cm SA, overlocked |
| Shrinkage | Max 3% warp, 3% weft after one wash |
| Weight (size L) | 210 g ±10 g finished |

### Crew-neck tee

| Fabric | Combed cotton 100%, single jersey, 180–200 GSM |
| --- | --- |
| Neck | 1×1 rib knit, 2.0 cm band, self-fabric color-matched |
| Neck tape | Tonal cotton twill tape across back neck |
| Stitching | Single-needle on neck binding; double-needle coverstitch on sleeve & bottom hems |
| Sleeve | Short, 3.0 cm double-needle hem |
| Side seam | Set-in tubular body OR side-seamed depending on manufacturer; confirm with vendor |
| Shrinkage | Max 4% warp, 4% weft after one wash |
| Weight (size L) | 180 g ±10 g finished |

### Finishing

- Pre-washed to control shrinkage before cut.

- All raw edges overlocked; no frayed edges in finished goods.

- Remove all thread tails; no knots visible on face side.

- Care label in Thai + English heat-transferred at left hip side seam.

# 4.  Tech pack — Polo

| Front · Classic concept (reference) | Back · Classic concept (reference) |
| --- | --- |

### Branding placement — Polo

| Element | Method | Size (W×H) | Placement anchor | Notes |
| --- | --- | --- | --- | --- |
| Chest-left logo | Embroidery | 70 × 70 mm | Center 90 mm below HPS¹, 100 mm from center-front placket | Logo variant per ink matrix. 9000-stitch max. |
| Big back logo | Embroidery | 170 × 170 mm | Center, 180 mm below HPS centerline | Concepts 02 + 04 only. Confirm density for dark shirts. |
| Sleeve URL | Screen print or embroidery | Text height 8 mm, length ~90 mm | Viewer-left sleeve, centered on sleeve hem band, 15° tilt toward cuff | All caps. Font: Inter Bold. |
| Inner-neck tagline | Heat-transfer label | 60 × 8 mm | Centered below collar band, 20 mm below neckline | Thin stretch film, color per ink matrix. |
| Hem tagline | Screen print | Text height 6 mm, length ~48 mm | Center-front, 45 mm above hem stitch | Concept 03 only. |
| Gradient stripe | Sublimation or screen | Length 280 mm, thickness 16 mm | Angled 22° across left shoulder, starting 10 mm from shoulder seam | Concept 03 only. Gradient teal→green→lime. On green shirts: solid white. |
| Chest wordmark | Screen print | Text height 26 mm, length ~220 mm | Center chest, 200 mm below HPS | Concept 04 only. Green dot at end (8 mm dia.). |
| Back stacked slogan | Screen print | Each line 55 mm high | Centered, starting 220 mm below HPS, 20 mm line gap | Concept 04 only. Keep period after 'MOVES.' |

¹ HPS = High Point of Shoulder. All measurements are finished-garment dimensions on a size L.

# 5.  Tech pack — Crew-neck tee

| Front · Classic concept (reference) | Back · Classic concept (reference) |
| --- | --- |

### Branding placement — Tee

Identical to polo placements except: chest-left logo sits 85 mm below HPS (the crew neck is higher than a polo's open collar). Hem tagline for concept 03 sits 35 mm above hem stitch. All other anchors match the polo table in section 4.

# 6.  Size chart

All measurements in centimeters, tolerance ±1.5 cm. Chest measured 2.5 cm below armpit, laid flat and doubled for full circumference.

### Polo — finished measurements

| Size | Chest (cm) | Body length (cm) | Shoulder (cm) | Sleeve length (cm) | Neck opening (cm) |
| --- | --- | --- | --- | --- | --- |
| XS | 96 | 66 | 41 | 20 | 38 |
| S | 100 | 68 | 43 | 21 | 39 |
| M | 104 | 70 | 45 | 22 | 40 |
| L | 110 | 72 | 47 | 23 | 41 |
| XL | 116 | 74 | 49 | 24 | 42 |
| 2XL | 122 | 76 | 51 | 25 | 43 |
| 3XL | 128 | 78 | 53 | 26 | 44 |

### Crew tee — finished measurements

| Size | Chest (cm) | Body length (cm) | Shoulder (cm) | Sleeve length (cm) | Neck opening (cm) |
| --- | --- | --- | --- | --- | --- |
| XS | 94 | 66 | 40 | 19 | 36 |
| S | 98 | 68 | 42 | 20 | 37 |
| M | 102 | 70 | 44 | 21 | 38 |
| L | 108 | 72 | 46 | 22 | 39 |
| XL | 114 | 74 | 48 | 23 | 40 |
| 2XL | 120 | 76 | 50 | 24 | 41 |
| 3XL | 126 | 78 | 52 | 25 | 42 |

# 7.  Decoration methods

### Embroidery (preferred for logos)

- Chest-left and big back logo variants. Thread: polyester; stitch density 4.0–4.4 pt.

- Stabilizer: medium-weight cut-away on knit fabric.

- Max stitch count: 9,000 (chest), 22,000 (back).

- Use approved DST file from brand owner — do not redigitize without approval.

### Screen print

- Preferred for text-heavy elements (wordmark, back slogan, hem tagline).

- Plastisol ink on knits; soft-hand additive for comfort.

- Mesh count: 305 for fine type; 156 for solid blocks.

- Cure temp 160°C for 90 s; snap-cure not accepted on production runs.

### Sublimation (gradient only)

- Concept 03 gradient stripe on polyester-blend (polo) only. 100% cotton tees cannot sublimate — use screen print with two-color simulated gradient instead.

- Color calibration must be confirmed against Pantone references (§2) before production.

### Heat-transfer label

- Inner-neck tagline only. 45 µm stretch film, matte finish.

- Apply at 160°C / 15 s / medium pressure. Peel hot.

# 8.  Design concept gallery

Reference mockups for each concept. Black colorway shown; other colors rendered per ink matrix (§2). Full 64-file mockup library is in 05-uniforms/mockups/.

### 01 · Classic

Chest-left logo + sleeve URL + inner-neck tagline. The everyday uniform.

| Polo · Deep Black | Tee · Deep Black |
| --- | --- |

### 02 · Bold Back

Small chest logo + large embroidered back logo + tagline. High-visibility option.

| Polo · Deep Black | Tee · Deep Black |
| --- | --- |

### 03 · Gradient

Signature 3-color gradient stripe on left shoulder + hem tagline. Brand signature.

| Polo · Deep Black | Tee · Deep Black |
| --- | --- |

### 04 · Typography

Big 'RAWINNIPA' wordmark on chest + stacked 'BUILD / WHAT / MOVES.' on back.

| Polo · Deep Black | Tee · Deep Black |
| --- | --- |

# 9.  Care & packaging

### Care instructions (label)

- Machine wash cold, inside out.

- Non-chlorine bleach only if needed.

- Tumble dry low; remove promptly.

- Do not iron directly on prints or embroidery.

- Do not dry clean.

### Packaging

- Individual flat-fold with collar support board (polo) or tissue paper (tee).

- Poly-bag: 40 µm LDPE, 300 × 380 mm, resealable.

- Hang tag: kraft card 250 gsm, 50 × 90 mm, black string loop, logo foil-stamped.

- Cartons: 50 pieces per master carton, single size per carton, label with SKU + size + color + quantity.

### Labeling

- Size label: woven, black with white text, at center-back neck.

- Care label: printed on satin, below size label.

- Country of origin label: joint side seam at left hip.

# 10.  QA & approval

### Pre-production samples

- Submit 1 sample per garment × per concept × per color = 32 samples for brand approval before bulk production.

- Each sample must include all four decoration types relevant to its concept.

- Turnaround target: 14 working days after PO confirmation.

### Inspection points

- Color match vs. Pantone reference (§2) — confirmed in natural daylight.

- Logo placement — measured from HPS per §4/§5 tables, ±5 mm tolerance.

- Logo size — measured W×H, ±2 mm tolerance.

- Embroidery quality — no puckering, no loose threads, registration within 0.5 mm.

- Stitch density — count per 10 mm² must match digitized file ±10%.

- Shrinkage — max per §3 after one standard wash cycle.

### Acceptance criteria

- AQL 2.5 for major defects, AQL 4.0 for minor, per ISO 2859-1.

- Any unit with logo misregistration >2 mm, wrong color thread, or print ghosting is rejected.

### Approval sign-off

| Role | Name | Date / signature |
| --- | --- | --- |
| Brand owner |  |  |
| Production manager |  |  |
| QA lead |  |  |

### Contact

All questions about this spec sheet: sayhi@rawinsoft.com